# Lesson 4 Student Workbook

Hello ESP32

Today you will move from desktop C++ to a real microcontroller. Your goal is to safely upload a first ESP32 sketch and see messages from the board in the Serial Monitor.

# Key Words

| Word | Meaning |
| --- | --- |
| ESP32 | A small microcontroller board that can run your code. |
| sketch | An Arduino program. |
| upload | Send your program from the laptop to the ESP32. |
| setup | Runs once when the board starts or resets. |
| loop | Runs again and again while the board has power. |
| Serial Monitor | A window that shows messages from the board. |

# Safety First

Use only the USB cable for power in this lesson.

Do not connect motors, batteries, sensors, or loose wires yet.

Place the ESP32 on a clean desk, not on metal.

Ask before pressing buttons or changing board settings if you are unsure.

# Starter Sketch

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("Hello, ESP32!");

Serial.println("Setup runs once.");

}

void loop() {

Serial.println("Loop is running.");

delay(1000);

}

# What the Sketch Does

| Code | Meaning |
| --- | --- |
| Serial.begin(115200); | Start Serial communication at 115200 baud. |
| Serial.println(...); | Print a line to the Serial Monitor. |
| delay(1000); | Wait about one second. |
| setup() | Runs once when the board starts. |
| loop() | Repeats while the board is powered. |

# Try It

Connect the ESP32 with a USB data cable.

Open Arduino IDE.

Select the ESP32 board and port.

Open the Lesson 4 starter sketch.

Click Upload.

Open Serial Monitor.

Set the Serial Monitor speed to 115200 baud.

Watch the messages from setup and loop.

# Reflection

What message printed only once? ___________________________________________

What message repeated? _________________________________________________

What is one difference between desktop C++ and ESP32 code? ______________