# Lesson 4 Teacher Lesson Plan

Level 1 C++ Foundations Hello ESP32

Lesson 4 is the first hardware lesson. Students connect an ESP32, upload a simple Arduino sketch, and use the Serial Monitor to observe setup and loop. No motors, sensors, or external wiring are used.

# Lesson Snapshot

| Item | Plan |
| --- | --- |
| Audience | Middle and high school robotics students |
| Length | 60 to 90 minutes |
| Prerequisite | Lessons 1 to 3 and a laptop with Arduino IDE plus ESP32 board support |
| Big idea | ESP32 programs still use C++ ideas, but Arduino sketches start with setup and loop instead of main. |
| Student product | A working ESP32 sketch that prints messages to the Serial Monitor |

# Learning Objectives

Identify the ESP32 as a microcontroller board.

Select an ESP32 board and port in Arduino IDE.

Explain the difference between setup and loop.

Upload a sketch to the ESP32.

Use Serial.begin and Serial.println to view board output.

Modify and re-upload a safe no-motion sketch.

# Materials Per Student Pair

One ESP32 DevKit-style board.

One USB data cable, not charge-only.

Laptop with Arduino IDE installed.

ESP32 board support installed through Boards Manager.

Student workbook and starter sketch.

# Setup Reference

| Item | Recommended Setting |
| --- | --- |
| Board Manager URL | https://espressif.github.io/arduino-esp32/package_esp32_index.json |
| Board package | esp32 by Espressif Systems |
| Common board choice | ESP32 Dev Module, unless your exact board is listed |
| Serial Monitor speed | 115200 baud |
| Safety rule | No external wiring, motors, or robot battery in this lesson |

# Timing Guide

| Time | Activity |
| --- | --- |
| 0 to 10 min | Review desktop C++ and introduce the ESP32 board. |
| 10 to 25 min | Check Arduino IDE, board package, board selection, and port. |
| 25 to 40 min | Teacher demo: upload Hello ESP32. |
| 40 to 60 min | Students upload, open Serial Monitor, and observe output. |
| 60 to 75 min | Students modify messages and re-upload. |
| 75 to 90 min | Troubleshooting, skill check, and exit response. |

# Starter Sketch

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("Hello, ESP32!");

Serial.println("Setup runs once.");

}

void loop() {

Serial.println("Loop is running.");

delay(1000);

}

# Teaching Notes

Keep the first hardware win small: board connected, sketch uploaded, Serial Monitor showing output.

Explain that Arduino hides main, but the board still runs C++ code.

Say setup runs once after reset; loop repeats until power is removed or a new sketch is uploaded.

If upload fails, check cable, port, board choice, and whether the BOOT button must be held during upload.